Section II: Schedule of Requirements

A.	Summary of Requirements

UNOPS requirements are comprised of six types of tractor Implements which will be operated with existing tractors. Each equipment are considered as a single lot.

Lot 1: Disc Plough

Lot 2: Disc Harrow

Lot 3: Ridger

Lot 4: Planter

Lot 5: Mounted chemical sprayer

Lot 6: Slasher, Mounted rotary slasher

The Implements are to be supplied and delivered to Ethiopian Institute of Agricultural Research (EIAR), Ethiopia.  Tools, parts and accessories should be supplied with the equipment. Manufacturer recommended standard accessories, tools and spare parts shall be listed and quoted separately. Please provide the recommendation of the manufacturer. Moreover, the equipment requires additional services like installation and warranty.

Technical specifications for Goods and Comparative Data Table

The following are the minimum requirements for the equipment

| Item No | UNOPS minimum technical requirements |  |  | Is Bid compliant? Bidder to complete |
| --- | --- | --- | --- | --- |
|  |  |  |  | This column must be completed fully by bidders |
| Lot 1 | Disc Plough |  | 16 |  |
|  | Type |  |  |  |
| 1 | 4 bottom, mounted disc plough |  |  | ☐ Yes ☐ No |
| 2 | Tapered roller bearings on discs and furrow wheel |  |  | ☐ Yes ☐ No |
| 3 | Adjustable vertical & horizontal angle of discs preferable |  |  | ☐ Yes ☐ No |
| 4 | Floating furrow wheel ensuring cutting angle adjustment & side draft control Options depth control wheel |  |  | ☐ Yes ☐ No |
| 5 | Fully mounted with three point linkage Cat 2 pin |  |  | ☐ Yes ☐ No |
| 6 | Adjustable scrapers for each bottom |  |  | ☐ Yes ☐ No |
| 7 | Disc diameter 660mm |  |  | ☐ Yes ☐ No |
| 8 | Plain disc, ISO Standard, not less than 4.5mm thickness preferably with vertical & horizontal angle disc adjustment |  |  | ☐ Yes ☐ No |
|  | Construction |  |  |  |
| 9 | Heavy duty rectangular or seamless tabular steel frame with parking stand |  |  | ☐ Yes ☐ No |
|  | Weights and Dimensions |  |  |  |
| 10 | Working width 98-105cm |  |  | ☐ Yes ☐ No |
| 11 | Ploughing depth 20-30cm |  |  | ☐ Yes ☐ No |
|  | Tractor Power |  |  | ☐ Yes ☐ No |
| 12 | Disc Plough must be operable by/compatible with 100-120 HP existing tractors |  |  | ☐ Yes ☐ No |
|  | Manual and Catalogues |  |  |  |
| 13 | Operators manual |  |  | ☐ Yes ☐ No |
| 14 | Repair manual |  |  | ☐ Yes ☐ No |
| 15 | Parts catalogue for each disc plough |  |  | ☐ Yes ☐ No |
|  | Other Requirement |  |  | ☐ Yes ☐ No |
| 16 | Assembly and demonstration on site will be handled by the supplier |  |  | ☐ Yes ☐ No |
| 17 | Tools (Manufacturer standard service tools) |  |  | ☐ Yes ☐ No |
| 18 | The bidder shall have manufacturer authorized local agent/dealer who can provide Pre-deliver Inspection , warranty and after sales service and spare parts in Ethiopia |  |  | ☐ Yes ☐ No |
| 19 | If No, can you commit/agree to arrange a local agent/dealer within 2-3 weeks after award |  |  | ☐ Yes ☐ No |
|  | Spare parts for Disc plough |  |  |  |
| 1.1 | Line pin top |  | 16 | ☐ Yes ☐ No |
| 1.2 | lower hitch pin |  | 16 | ☐ Yes ☐ No |
| 1.3 | bearing inner |  | 64 | ☐ Yes ☐ No |
| 1.4 | bearing outer |  | 64 | ☐ Yes ☐ No |
| 1.5 | Disc diameter 660 |  | 64 | ☐ Yes ☐ No |
| 1.6 | Furrow wheel disc |  | 16 | ☐ Yes ☐ No |
| 1.7 | blade bolt with nut |  | 64 | ☐ Yes ☐ No |
| 1.8 | spring |  | 16 | ☐ Yes ☐ No |
| Lot 2 | DISC Harrow |  | 16 |  |
| 1 | Type | Trailed Disc harrow( hydraulically lifted) |  | ☐ Yes ☐ No |
| 2 |  | Three point Hitch |  | ☐ Yes ☐ No |
| 3 | Structure | Heavy duty Special steel frame |  | ☐ Yes ☐ No |
| 4 | Bearings | Conical grease or oil bearings |  | ☐ Yes ☐ No |
| 5 | Disc type | Front: Notched (Scalloped) disc |  | ☐ Yes ☐ No |
| 6 |  | Rear: Plain disc and set a two adjustable off set gangs |  | ☐ Yes ☐ No |
| 7 | Number of discs | 28 discs |  | ☐ Yes ☐ No |
| 8 | Disc Diameter | 610 mm |  | ☐ Yes ☐ No |
| 9 | Disc spacing | 185-300 mm |  | ☐ Yes ☐ No |
| 10 | Disc Thickness | Not less than 4.5mm |  | ☐ Yes ☐ No |
| 11 | Axle Diameter | Manufacturer’s standard (square high carbon steel preferable) |  | ☐ Yes ☐ No |
| 12 | Gang Angle | 0-45 degree or manufacturer standard |  | ☐ Yes ☐ No |
| 13 | Scarpers | Paddle type fitted and manually adjustable |  | ☐ Yes ☐ No |
| 14 | Working Width | 265- 400 |  | ☐ Yes ☐ No |
| 15 | Working depth | 150-250mm (15-25cm) |  | ☐ Yes ☐ No |
| 16 | Tractor Compatibility | Disc Harrow must be operable by/compatible with 100-120HP existing tractors |  | ☐ Yes ☐ No |
| 17 | Tools | Manufacturer standard service tools & gang axle nut spanners |  | ☐ Yes ☐ No |
| 18 | Manuals and catalogues | Operators & repair manual |  | ☐ Yes ☐ No |
| 19 |  | Parts catalogue for each disc harrow |  | ☐ Yes ☐ No |
| 20 | Other Requirement |  |  |  |
| 21 | Assembly and demonstration on site will be handled by the supplier |  |  | ☐ Yes ☐ No |
| 22 | The bidder shall have manufacturer authorized local agent/dealer who can provide Pre-deliver Inspection , warranty and after sales service and spare parts in Ethiopia |  |  | ☐ Yes ☐ No |
| 23 | If No, can you commit/agree to arrange a local agent/dealer within 2-3 weeks after award |  |  | ☐ Yes ☐ No |
|  | Spare parts for Disc Harrow |  |  |  |
| 1.1 | Scalloped/notched disc |  | 112 | ☐ Yes ☐ No |
| 1.2 | Plain disc |  | 112 | ☐ Yes ☐ No |
| 1.3 | Hunger bearing |  | 40 | ☐ Yes ☐ No |
| 1.4 | Tire bearings inner and outer |  | 32 | ☐ Yes ☐ No |
| 1.5 | Different bolts and nuts |  | 128 | ☐ Yes ☐ No |
| 1.6 | Hitch Pin |  | 48 | ☐ Yes ☐ No |
| 1.7 | Hydraulic Hose set |  | 16 |  |
| Lot 3 | Ridger |  | 16 |  |
| 1 | Type | Mounted Riger with adjustable wings and tine positioning as to crops |  | ☐ Yes ☐ No |
| 2 |  | Three point hitch |  | ☐ Yes ☐ No |
| 3 |  | Preferable Accessories: gauge/depth control wheels, universal fore carriage for pull-type conversion |  | ☐ Yes ☐ No |
| 4 | Construction | Tubular or rectangular steel bar with safety bolts |  | ☐ Yes ☐ No |
| 5 |  | Tubular bar Length 420cm or better |  | ☐ Yes ☐ No |
| 6 | No of Bottoms | 4 rows |  | ☐ Yes ☐ No |
| 7 | Row Spacing | 120/150 cm |  | ☐ Yes ☐ No |
| 8 | Working Depth | 20-30cm |  | ☐ Yes ☐ No |
| 9 | Tractor Compatibility | Ridger must be operable by/compatible with 100-120 HP existing tractors |  | ☐ Yes ☐ No |
| 10 | Spare Parts | Manufacturer’s recommended spare parts for two years operations |  | ☐ Yes ☐ No |
| 11 | Tools | Manufacturer standard service tools |  | ☐ Yes ☐ No |
| 12 | Manuals & Catalogue | Operators manual |  | ☐ Yes ☐ No |
| 13 |  | Parts catalogue |  | ☐ Yes ☐ No |
| 14 | Other requirements | should have after sales service and spare parts in Ethiopia |  | ☐ Yes ☐ No |
| 15 |  | Assembly and demonstration on site will be handled by the supplier |  | ☐ Yes ☐ No |
| 16 | The bidder shall have manufacturer authorized local agent/dealer who can provide Pre-deliver Inspection , warranty and after sales service and spare parts in Ethiopia |  |  | ☐ Yes ☐ No |
| 17 | If No, can you commit/agree to arrange a local agent/dealer within 2-3 weeks after award |  |  | ☐ Yes ☐ No |
| Lot 4 | Planter |  | 16 |  |
| 1 | Type | Mounted precision & pneumatic row crop planter with fertilizer hopper attachment |  | ☐ Yes ☐ No |
| 2 |  | Purpose: for all types of seeds including (cereals, pulses, oil and forage crops on conventional seed beds) |  | ☐ Yes ☐ No |
| 3 |  | Gear box with combinations to calibrate the seed & fertilizer distribution |  | ☐ Yes ☐ No |
| 4 | Construction | Hard steel |  | ☐ Yes ☐ No |
| 5 | No Of Rows | 4 rows |  | ☐ Yes ☐ No |
| 6 | Row Spacing | 40 -80cm (adjustable) |  | ☐ Yes ☐ No |
| 7 | Seed Hopper Capacity | 45L or better |  | ☐ Yes ☐ No |
| 8 | Fertilizer Hopper Capacity | 320 - 560L |  | ☐ Yes ☐ No |
| 9 | PTO | 540rpm |  | ☐ Yes ☐ No |
| 10 | Lifting Capacity | 2000-3000kg |  | ☐ Yes ☐ No |
| 11 | Weight | Weight: 800-1000kg total |  | ☐ Yes ☐ No |
| 14 | Working Depth | 0 -120mm |  | ☐ Yes ☐ No |
| 15 | Tractor Compatibility | Planter must be operable by/compatible with 100-120 HP existing tractors |  | ☐ Yes ☐ No |
| 16 | Wheel & Tyres | Manufacturer standard tire with inner tube |  | ☐ Yes ☐ No |
| 17 | Spare Parts | Manufacturer’s recommended spare parts for two years operations |  | ☐ Yes ☐ No |
| 18 | Tools | Manufacturer standard service tools |  | ☐ Yes ☐ No |
| 19 | Manuals & Catalogue | Operators and repair manual |  | ☐ Yes ☐ No |
| 20 |  | Parts catalogue |  | ☐ Yes ☐ No |
| 21 | Other requirements | Assembly and demonstration on site will be handled by the supplier |  | ☐ Yes ☐ No |
| 22 | Warranty | A period of 12 months after delivery |  | ☐ Yes ☐ No |
| 24 | The bidder shall have manufacturer authorized local agent/dealer who can provide Pre-deliver Inspection , warranty and after sales service and spare parts in Ethiopia |  |  | ☐ Yes ☐ No |
| 25 | If No, can you commit/agree to arrange a local agent/dealer within 2-3 weeks after award |  |  | ☐ Yes ☐ No |
| Lot 5 | Mounted Chemical Sprayer |  | 16 |  |
| 1 | Type | Fully hydraulic boom sprayer, easily operated & maintainable, strongly built & Quality sprayer |  | ☒ Yes ☐ No |
| 2 |  | 4 Ways pressure control, Master flow/VAR co valve or better |  | ☐ Yes ☐ No |
| 3 | Pressure Gauge | Extended scale 0 – 500psi |  | ☐ Yes ☐ No |
| 4 | Inline Filter | Quantity: 4, Mesh: 80 |  | ☐ Yes ☐ No |
| 5 | Monojet Single Nozzle Holder | Equipped with non-drip valve and installed on hose for easy adjustment of nozzle spacing, ceramic |  | ☐ Yes ☐ No |
| 6 |  | Spacing: 50cm, Quantity: 25-29 |  | ☐ Yes ☐ No |
| 10 | Tank | Capacity: 600 litres or better |  | ☐ Yes ☐ No |
| 11 |  | Tank: made of high density polyethylene that is filtrates the ultraviolet rays, it also chemical corrosion & impacts resistant It is equipped with level indicator and strainer |  | ☐ Yes ☐ No |
| 12 |  | Extra 12 liter tank for the operator protection and comfort |  | ☐ Yes ☐ No |
| 13 | Filter | Mesh: 60 |  | ☐ Yes ☐ No |
| 14 | Pump | Type: three pistons with ceramic liners that resist chemicals abrasive action |  | ☐ Yes ☐ No |
| 15 |  | Capacity: 75L/min or better |  | ☐ Yes ☐ No |
| 16 |  | Maximum working pressure: 150-200psi |  | ☐ Yes ☐ No |
| 17 |  | Maximum power consumption: 22-30HP |  | ☐ Yes ☐ No |
| 18 | Boom | Cross-folding system the sprayer to be easily transported or better |  | ☐ Yes ☐ No |
| 19 |  | Length: 12-14m, Actuation: hydraulic or better |  | ☐ Yes ☐ No |
| 20 |  | Height: 0.50-1.30m or better |  | ☐ Yes ☐ No |
| 21 | Agitator | Through hydraulic return or better |  | ☐ Yes ☐ No |
| 22 | Level Indicator | Graduated hose attached to the tank or better |  | ☒ Yes ☐ No |
| 23 | Tractor Compatibility | sprayer must be operable by/compatible with 100-120 HP existing tractors |  | ☐ Yes ☐ No |
| 24 | Spare Parts | Manufacturer’s recommended spare parts for two years operations |  | ☐ Yes ☐ No |
| 25 | Tools | Manufacturer standard service tools |  | ☐ Yes ☐ No |
| 26 | Manuals & Catalogue | Operators and repair manual |  | ☐ Yes ☐ No |
| 27 |  | Parts catalogue |  | ☐ Yes ☐ No |
| 28 | Other requirements | Assembly and demonstration on site will be handled by the supplier |  | ☐ Yes ☐ No |
| 30 | Warranty | A period of 12 months after delivery |  | ☐ Yes ☐ No |
| 31 | The bidder shall have manufacturer authorized local agent/dealer who can provide Pre-deliver Inspection , warranty and after sales service and spare parts in Ethiopia |  |  | ☐ Yes ☐ No |
| 32 | If No, can you commit/agree to arrange a local agent/dealer within 2-3 weeks after award |  |  | ☐ Yes ☐ No |
| Lot 6 | Slasher , mounted rotary slasher |  | 16 |  |
| 1 | Type | Mounted rotary slasher used for cutting grass, maize stalks, bush and clearing farms |  | ☐ Yes ☐ No |
| 2 |  | Auxiliary linkage, central and lateral, side plates with replaceable foils and depth control rear wheel |  | ☐ Yes ☐ No |
| 3 |  | Cardan shaft with safety clutch |  | ☐ Yes ☐ No |
| 4 |  | Open able rear cover |  | ☐ Yes ☐ No |
| 5 | Construction | Heavy duty structure reinforced frame, hard steel |  | ☐ Yes ☐ No |
| 6 | Blades | Reversible, double edge |  | ☐ Yes ☐ No |
| 7 |  | Forged & heat treated from special steel for a long service life, while working in hard conditions |  | ☐ Yes ☐ No |
| 8 |  | No of blades – 2 |  |  |
| 9 |  | Peripheral speed of cutting blades up to 4 7 m/min |  | ☐ Yes ☐ No |
| 10 | Hitching | 3 point linkage cat I & II |  | ☐ Yes ☐ No |
| 11 |  | Upper hitch swinging type |  | ☐ Yes ☐ No |
| 12 | Clutch | Multiple clutch to protect the transmission |  | ☐ Yes ☐ No |
| 13 |  | Clutch with friction disc |  | ☐ Yes ☐ No |
| 14 | Weight & Dimensions | Cutting width: 170-200 cm |  | ☐ Yes ☐ No |
| 15 |  | Cutting height: : 5 -30 cm |  | ☐ Yes ☐ No |
| 16 |  | Weight: 400 – 850kg |  |  |
| 18 | Rpm On PTO | 540 rpm |  | ☐ Yes ☐ No |
| 19 | Tractor Compatibility | Slasher must be operable by/compatible with 100-120 HP existing tractors |  | ☐ Yes ☐ No |
| 20 | Tools | Manufacturer standard service tools |  | ☐ Yes ☐ No |
| 21 | Manuals & Catalogue | Operators manual |  | ☐ Yes ☐ No |
| 22 |  | Parts catalogue |  | ☐ Yes ☐ No |
| 23 | Other requirements | Assembly and demonstration on site will be handled by the supplier |  | ☐ Yes ☐ No |
| 25 | The bidder shall have manufacturer authorized local agent/dealer who can provide Pre-deliver Inspection , warranty and after sales service and spare parts in Ethiopia |  |  | ☐ Yes ☐ No |
| 26 | If No, can you commit/agree to arrange a local agent/dealer within 2-3 weeks after award |  |  | ☐ Yes ☐ No |
|  | Spare parts for Slasher |  |  |  |
| 1.1 | Hitch Pin |  | 64 | ☐ Yes ☐ No |
| 1.2 | Bearing |  | 64 | ☐ Yes ☐ No |
| 1.3 | Hub Screw |  | 32 | ☐ Yes ☐ No |
| 1.4 | Cutting edge/blade |  | 32 | ☐ Yes ☐ No |
| 1.5 | Blade screw |  | 32 | ☐ Yes ☐ No |
| 1.6 | Blade Nut |  | 32 | ☐ Yes ☐ No |
| 1.7 | Hub Nut |  | 32 | ☐ Yes ☐ No |

Note: Bidders shall bear in mind that there are criteria in the above schedule of requirements to be completed /filled by writing the required response other than the Yes/No response.

- Bidders are required to clearly complete the detailed technical criteria for each proposed product.  The specification of the proposed product must correspond with the UNOPS minimum requirements indicated in part B of this document (Technical specifications for Goods and Comparative Data Table) i.e. for each product requirement, the bidder must provide the corresponding specification demonstrating alignment with the specified minimum technical requirement.   In the case of deviation, the bidder is to provide an additional explanation in the remarks column.

- The Bidder’s specification shall be consistent with the brand and model number indicated in the product data sheet /catalogue submitted. Bidders are requested to provide manufacturer’s product data sheet /catalogue to support the bidder’s proposed product specification. Failure to provide the proposed products’ technical details in Section III- returnable bidding form (Form G) may result in the disqualification of the bid

- For each item, bidder must propose/indicate only one Brand, Model and Model no. Proposing/indicating more than one Brand, Model and Model is forbidden.

- Please provide manufacturer standard list and quantities of tools , accessories and parts for the equipment that require recommend lists (and quote for these tools , accessories  and parts on form F)

Additional required information that bidders are expected to provide together with their bid submission.

- Manufacturer name and year of manufacture of the chosen equipment

- Country of origin of the chosen equipment

- Detailed product data sheet/ catalogue of the chosen equipment

- If a Bidder does not manufacture or produce the goods it offers to supply, he shall submit the Manufacturer’s Authorization using the form included in Section III: Returnable Bidding Forms to demonstrate that it has been duly authorized by the manufacturer or producer of the goods to supply these goods in the country of destination

Additional Notes: The bidder shall have a local agent who have workshops and can provide spare parts/tools for the quoted equipment. The local agent shall provide aftersales services like, repair, maintenance and warranty services. Please provide name and contact details of the local agent in Ethiopia

- Manufacturer website if possible

- Delivery requirements and Comparative Data Table.

- The delivery requirement stated below is applicable for all items.

- Bidders should also complete the below table and insert in to the form G (Technical bid form) for submission.

- Bidders should also state that the delivery requirement they have completed and inserted in the form G (technical bid form) using the below table is ‘’ applicable for all the items ‘’ they have quoted for.

| UNOPS Requirements |  | Is Bid compliant? Bidder to complete | Details Bidder to complete |
| --- | --- | --- | --- |
| Delivery time | Bidder shall deliver all/ quoted equipment to the partner within 150days after Contract signature. | ☐ Yes ☐ No | Insert details |
| Delivery place and Incoterms rules | DAP Addis Ababa, Ethiopia including custom clearance handling , as per Incoterms 2010 | ☐ Yes ☐ No | Insert details |
| Consignee details | Federal Democratic Republic of Ethiopia, Ethiopian Institute of Agricultural Research, Addis Ababa, Ethiopia. | ☐ Yes ☐ No | Insert details |
| UNOPS Right to vary requirements | At the time the Contract is awarded, UNOPS reserves the right to vary the quantity of the goods and associated services specified above, provided this does not exceed +/- 20%, without any change in the unit prices or other terms and conditions of the ITB. | ☐ Yes ☐ No | Insert details |

D.	 Inspections and tests

The following inspections and tests shall be performed:

The vendor must have the goods inspected in the manufacturer’s works by a competent authority and submit a test certificate and also a guarantee/warranty certificate that the goods conform to written specifications.

UNOPS or its representative may decide to inspect and/or test any or all item of the goods to confirm their conformity to the contract, prior to dispatch from the manufacturer’s premises. Such inspection and clearance will not prejudice the right of the consignee to inspect and test the goods on receipt at destination.

If the goods fail to meet the laid down specifications, the supplier shall take immediate steps to remedy the deficiency or replace the defective goods to the satisfaction of the purchaser.